| Python Code | Annotations |
| --- | --- |
| import pandas as pd import numpy as np import matplotlib.pyplot as plt import seaborn as sns # Set a clean visual style for all our plots sns.set_style('whitegrid') sns.set_palette('deep') | Imports the main analysis libraries: pandas for tabular data, numpy for numerical work, matplotlib and seaborn for plotting. The last two lines set a consistent seaborn theme and color palette for the charts. |
| ngsim_data = pd.read_csv('NGSIM.csv') | Loads the NGSIM traffic dataset from a CSV file into the dataframe named ngsim_data so it can be analyzed later. |
| nhts_data = pd.read_csv('NHTS.csv') | Loads the NHTS dataset from a CSV file into the dataframe named nhts_data. |
| # Simple bar chart: count of vehicles by fuel type # First we need to count the values, then plot census_region = nhts_data['census_region'].value_counts().dropna() print(census_region) ## Step 1: count the number of vehicles by fuel type # hint: use .value_counts() plt.figure(figsize = (12,6)) census_region.plot(kind = 'bar', color = sns.color_palette('viridis', len(census_region)), edgecolor = 'black') ## Step 2: create the bar plot plt.title('Number of Households by Census Region') plt.xlabel('Census Region') plt.ylabel('Count') ## Step 3: add labels and customization ## Step 4: show the plot plt.show() | Counts how many records fall into each census region, prints those counts, and then creates a bar chart. The comments mention fuel type, but the actual code is plotting census_region. |
| vehicles_urban = nhts_data[nhts_data['household_location'] == 'Urban']['vehicles_per_household'] plt.hist(vehicles_urban, bins = 20, edgecolor = 'black', alpha = 0.5, color='steelblue', label = 'Urban') ## Step 3: filter and plot Rural vehicle ages on the same figure vehicles_rural = nhts_data[nhts_data['household_location'] == 'Rural']['vehicles_per_household'] plt.hist(vehicles_rural, bins = 20, edgecolor = 'black', alpha = 0.5, color='coral', label = 'Rural') ## Step 4: add labels, title, and legend plt.title('Distribution of Vehicles per Household', fontsize=16) plt.xlabel('Number of Vehicles', fontsize = 12) plt.ylabel('Frequency', fontsize =12) plt.legend() plt.show() | Filters the dataset into Urban and Rural groups using household_location, then overlays two histograms of vehicles_per_household on the same figure. It finishes by adding a title, axis labels, a legend, and displaying the plot. |
| plt.figure(figsize = (14,6)) ## Step 2: call the type of plot and tell the function what variables you want to plot boxplot = nhts_data.boxplot(column = 'vehicle_age', by = 'urban') ## Step 3: add customization to the plot ### this line rotates your labels on the x-axis by 90 so they are easy to read plt.xticks(rotation = 90) ### the next two lines create your axis labels plt.xlabel('Urban Vehicles', fontsize = 12) plt.ylabel('Vehicle age (years)', fontsize = 12) ### this line gives your plot a title plt.title('Boxplot of Vehicle Age by Urban Area', fontsize = 14) ## Step 4: tell python to show the plot plt.show() | Creates a box plot of vehicle_age grouped by the urban field, then formats the x-axis labels and titles. This helps compare the distribution of vehicle age across categories. |
| # Step 1: tell python that you want to plot and assign a size plt.figure(figsize=(12,5)) # Step 2: use sns.lineplot to create a time series plot of Time (x-axis) and follower_acc(m/s^2) (y-axis) sns.lineplot(x = ngsim_data['Time'], y = ngsim_data['leader_speed(m/s)']) # Step 3: add customizations plt.xlabel('Time (seconds)', fontsize = 12) plt.ylabel('Speed (m/s)', fontsize = 12) plt.title('Leader Speed vs. Time', fontsize = 14) plt.show() | Builds a seaborn line plot of leader speed versus time from the NGSIM dataset. The labels and title are then added so the time-series plot is readable. |

| Python Code | Annotations |
| --- | --- |
| ### Plotting Acceleration vs Time # We are interested in analyzing the acceleration profile of the follower vehicles (follower_acc(m/s^2)) as function of time. Follow the steps we have learned from previous lab to do so. # hint we need to use the sns.lineplot # Step 1: tell python that you want to plot and assign a size plt.figure(figsize=(12,5)) # Step 2: use sns.lineplot to create a time series plot of Time (x-axis) and follower_acc(m/s^2) (y-axis) sns.lineplot(x = ngsim_data['Time'], y = ngsim_data['follower_speed(m/s)']) # Step 3: add customizations plt.xlabel('Time (seconds)', fontsize = 12) plt.ylabel('Speed (m/s)', fontsize = 12) plt.title('Follower Speed vs. Time', fontsize = 14) plt.show() | Creates a second time-series line plot using follower speed against time. The section header says acceleration, but the actual y-variable in the code is follower_speed(m/s). |
| # Define the IDM acceleration function # The function should take the following inputs: # v : current speed of the follower vehicle (m/s) # s : current gap distance between leader and follower (m) # delta_v : speed difference = follower speed - leader speed (m/s) # v0 : desired velocity (m/s) # s0 : minimum spacing (m) # T : desired time headway (s) # a : maximum acceleration (m/s^2) # b : comfortable deceleration (m/s^2) # delta : acceleration exponent def idm_acceleration(v, s, delta_v, v0, s0, T, a, b, delta): # Step 1: Calculate the desired minimum gap s_star s_star = s0 + v * T + (v * delta_v) / (2 * np.sqrt(a * b)) # Make sure s_star is not negative s_star = max(s_star, s0) # Step 2: Calculate the acceleration using the IDM equation acceleration = a * (1 - (v / v0)**delta - (s_star / s)**2) return acceleration | Defines the Intelligent Driver Model acceleration function. It computes the desired spacing s_star, prevents that value from dropping below the minimum spacing, and then returns the IDM acceleration using the model equation. |
| # Test the IDM function with sample values # A follower traveling at 20 m/s, with a gap of 25 m, approaching the leader at 2 m/s # Define parameters v0 = 30 # desired velocity (m/s) s0 = 2 # minimum spacing (m) T = 1.5 # desired time headway (s) a = 1.0 # maximum acceleration (m/s^2) b = 1.5 # comfortable deceleration (m/s^2) delta = 4 # acceleration exponent # Test case test_v = 20 # follower speed test_s = 25 # gap distance test_delta_v = 2 # follower is faster than leader by 2 m/s # Call the function and print the result test_acc = idm_acceleration(test_v, test_s, test_delta_v, v0, s0, T, a, b, delta) print(f"IDM Acceleration: {test_acc:.4f} m/s2") | Sets sample IDM parameters and a test case, calls the idm_acceleration function, and prints the resulting acceleration. This is a quick correctness check before running the full simulation. |
| # Load the NGSIM data to get the leader vehicle trajectory ngsim_data = pd.read_csv('NGSIM.csv') # Select a trajectory pair trajectory_number = 1 data_subset = ngsim_data[ngsim_data['trajectory_number'] == trajectory_number].reset_index(drop=True) # Extract the leader's position and speed over time time_data = data_subset['Time'].values leader_position = data_subset['leader_position(m)'].values leader_speed = data_subset['leader_speed(m/s)'].values # Identifying the time step in our data dt = 0.1 #every 0.1 seconds, I will calculate the acceleration value | Reloads the NGSIM data, isolates one trajectory pair, extracts time, leader position, and leader speed as arrays, and defines the simulation time step dt = 0.1 seconds. |
| # Set the IDM parameters v0 = 30 # desired velocity (m/s) s0 = 2 # minimum spacing (m) T = 1.5 # desired time headway (s) a_param = 1.0 # maximum acceleration (m/s^2) b = 1.5 # comfortable deceleration (m/s^2) delta = 4 # acceleration exponent # Initialize arrays to store simulation results n_steps = len(time_data) # number of simulation steps we will take, which will be equal to the length of the trajectory data sim_position = np.zeros(n_steps) # simulated follower position; we will create a vector with all zeroes as a start sim_speed = np.zeros(n_steps) # simulated follower speed; we will create a vector with all zeroes as a start sim_acc = np.zeros(n_steps) # simulated follower acceleration; we will create a vector with all zeroes as a start # Start at the real follower's initial state sim_position[0] = data_subset['follower_position(m)'].values[0] #this makes the starting value [0] same as the follower vehicle position sim_speed[0] = data_subset['follower_speed(m/s)'].values[0] | Defines the IDM parameters for the simulation, allocates numpy arrays for simulated position, speed, and acceleration, and seeds the first simulated follower position with the measured follower position from the dataset. |
| # Run the IDM simulation # We loop through each time step and calculate: # 1. The gap between the leader and follower # 2. The speed difference between the two vehicles # 3. The IDM acceleration # 4. The updated speed using: new_speed = old_speed + acceleration * dt # 5. The updated position using: new_position = old_position + speed * dt for i in range(n_steps - 1): # we do -1 since the first element is known (remember the step above) # Step 1 & 2: Calculate the gap and speed difference at this time step gap = max(leader_position[i] - sim_position[i], 0.1) # just a condition for gap not to be negative delta_v = sim_speed[i] - leader_speed[i] # Step 3: Use the IDM function to calculate acceleration sim_acc[i] = idm_acceleration(sim_speed[i], gap, delta_v, v0, s0, T, a_param, b, delta) # Step 4 & 5: Update speed and position for the next time step sim_speed[i+1] = max(sim_speed[i] + sim_acc[i] * dt, 0) # this updates my speed for next iteration based on physics equation of motion sim_position[i+1] = sim_position[i] + sim_speed[i] * dt print(i) print("Simulation complete!") | Runs the IDM simulation loop. For each time step it computes the gap and relative speed, calculates acceleration with the IDM function, updates the next speed and position, prints the loop index, and finally reports that the simulation is complete. |
| # Plot 1: Position vs Time — Comparing leader, real follower, and simulated follower plt.figure(figsize = (12,6)) plt.plot(time_data, data_subset['leader_acc(m/s^2)'], 'k--', linewidth = 2, label = 'Leader (NGSIM)') # plots leader position vs. time plt.plot(time_data, data_subset['follower_acc(m/s^2)'].values, 'b-', linewidth = 2, label = "Real Follower (NGSIM)")# this plots the actual follower position in my NGSIM data plt.plot(time_data, sim_acc, 'r-', linewidth = 2, label = 'Simulated Follower (IDM)') plt.xlabel('Time (seconds)') plt.ylabel('Acceleration (m/s^2)') plt.legend() plt.grid(True, alpha = 0.5) plt.show() | Plots acceleration versus time for the leader, the real follower, and the simulated follower on one graph. The opening comment says position vs time, but the plotted variables and y-axis label are acceleration values. |